Module 7: CCNA - Routing and switching

1.  Which of the following messages in the DHCP process are broadcasted? (Choose two)

A. Request

B. Offer

C. Discover

D. Acknowledge

Ans: A. Request     C. Discover

2.  Which command would you use to ensure that an ACL does not block web-based TCP traffic?

A. permit any

B. permit tcp any any eq 80

C. permit tcp any eq 80

D. permit any any eq tcp

Ans: B. permit tcp any any eq 80

3. Explain Network Topologies?

Ans: Network topology means how computers and devices are connected in a network    and how they communicate with each other.

There are two types of Network Topologies are there:

- Physical Topologies

- Logical Topologies

- Point – to – Point Topology

- Bus Topology

- Ring Topology

- Star Topology

- Mesh Topology

- Hybrid Topology

Physical Topologies

- In this topology, we can see the actual connection of devices using cables and wires. It shows how devices are physically connected to each other.

Logical Topology

- In this topology it shows how the process is done between devices how they communicate, share resources like files, folders and all.

Point – to – Point Topology

In this topology, there are only two devices connected directly. One device sends data and the other receives it. Communication can happen in both directions.

Bus Topology

In bus topology, all devices are connected to one main cable called backbone cable.

- If the backbone cable fails, the whole network stops working.

- There are terminators at both ends of the cable.

- It is mostly used for small networks.

Ring Topology

In this topology, devices are connected in a circular form.

- Data travels in one direction using a token system.

- Each device passes data to the next device.

- If one device or cable fails, the whole network can stop working.

Star Topology

In star topology, all devices are connected to a central device like a hub or switch.

- Devices communicate through this central device.

- If one cable fails, only that device is affected.

- But if the central device fails, the whole network stops.

Mesh Topology

In mesh topology, devices are connected with multiple connections.

- In full mesh, all devices are connected to each other.

- In partial mesh, only some devices are connected.

Hybrid Topology

Hybrid topology is a combination of two or more topologies.
It is mainly used in large networks.

- Explain TCP/IP Networking Model?

Ans: The TCP/IP Networking Model is a set of rules and protocols used for communication over the internet and in computer networks. It defines how data is transmitted from one device to another.

TCP/IP has 4 layers, which are given below:

- Application Layer
This layer provides services to user applications such as web browsers, email, and file transfer. It combines the functions of Application, Presentation, and Session layers of the OSI model.
Example: HTTP, HTTPS, FTP, SMTP, DNS

- Transport Layer
This layer is responsible for end-to-end communication and error-free data delivery. It divides data into segments and ensures proper transmission.
Example: TCP (reliable) and UDP (faster but less reliable)

- Internet Layer
This layer is used for logical addressing and routing of data packets from source to destination.
Example: IP, ICMP, ARP

- Network Access Layer
This layer is responsible for transmitting data over the physical network. It handles hardware addressing and communication with network devices.
Example: Ethernet, Wi-Fi

So, the TCP/IP model helps devices communicate properly by dividing networking tasks into these 4 layers.

- Explain LAN and WAN Network

Ans: LAN

LAN is a Local Area Network that covers small geographical area such as office, home, or school. It is used to connect computers and devices within a limited area. It can be wired--Ethernet or wireless—Wifi.

WAN

WAN is a Wide Area Network that covers large geographical area such as cities, countries. It is used to connect multiple LANs together. It is accessed by everyone we can say “Internet”.

- Explain Operation of Switch

Ans: Switch

A switch is a networking device used to connect multiple devices such as computers, printers, and servers in the same network. It works as a communication device and helps in sharing resources between connected devices.

A switch uses MAC addresses to send data to the correct device. When data comes to the switch, it checks the destination MAC address and forwards the data only to that specific device. This makes communication faster and more secure.

A switch maintains a MAC address table in which it stores the MAC addresses of all connected devices and the port numbers. This helps the switch to quickly identify where to send the data.

A switch also works as a central device in Star Topology, where all devices are connected to one central switch.

So, the main function of a switch is to provide efficient communication and proper data transfer within a network.

- Describe the purpose and functions of various network devices

Ans: Network Devices

Network devices are hardware components that help computers and other devices communicate with each other in a network. Each device has its own purpose and function.

- Router
A router is used to connect two or more different networks. It sends data from one network to another and helps connect a local network to the internet.

- Switch
A switch is used to connect multiple devices like computers, printers, and servers within the same network. It sends data only to the specific device for which it is meant.

- Hub
A hub also connects multiple devices in a network, but it sends data to all connected devices. It is less secure and slower than a switch.

- Modem
A modem connects a computer or router to the internet. It converts digital signals into analog signals and vice versa.

- Access Point (AP)
An access point provides wireless connectivity to devices like mobile phones and laptops in a network.

- Repeater
A repeater is used to strengthen or regenerate weak signals so they can travel longer distances.

- Bridge
A bridge connects two similar networks and helps reduce network traffic by filtering data.

- Gateway
A gateway connects networks that use different communication protocols and allows them to communicate.

- Firewall
A firewall is a security device that monitors and controls incoming and outgoing network traffic to protect the network from unauthorized access.

- NIC (Network Interface Card)
A NIC is a hardware component installed in a device to connect it to a network.

- Make list of the appropriate media, cables, ports, and connectors to connect switches to another device.

Ans: To connect a switch to another device, different types of media, cables, ports, and connectors are used depending on the device and network requirement.

1. Media:
Media is the path through which data is transferred.

- Wired Media

- Wireless Media

2. Cables:
Different cables are used for connecting a switch:

- Straight-through cable – Used to connect switch to PC, router, or server.

- Crossover cable – Used to connect switch to switch or switch to hub.

- Fiber Optic cable – Used for high-speed and long-distance connections.

- Console cable – Used to configure the switch.

3. Ports:
Ports available on the switch or other devices are:

- Ethernet port

- Fast Ethernet port

- Gigabit Ethernet port

- Console port

4. Connectors:
Connectors are attached to cables for connection.

- RJ45 – Used with Ethernet cables

- USB or Serial connector – Used with console cable

These media, cables, ports, and connectors are used to connect a switch with other networking devices properly.

- Define Network devices and hosts

Ans:  A network device is a hardware device that is used to connect computers and other electronic devices in a network. It helps in communication, data transfer, and managing the network. Examples of network devices are router, switch, hub, modem, and access point.

A host is any device connected to a network that can send or receive data. It is also called an end device. Examples of hosts are computer, laptop, mobile phone, printer, and server.